Assessing the use of unmodified 40-mer oligonucleotides in barcode microarray technology.

Choi, D. H. and Campbell, A. M.  Fall 2003. Davidson College, NC.

Independent Research.

# Abstract

Molecular barcodes (TAGs) have been employed in large scale yeast gene deletion studies.  The researchers have used commercially produced microarray chips that synthesize barcode oligonuleotides on the surface of the glass slide, such as GeneChip® arrays (Affymetrix, Inc.) in previous studies (Winzeler et al., 1999).  However, using unmodified oligonucleotides as the printing material on microarrays has not been common.  We present a pilot study that uses 96 samples of unmodified 40-mer oligonucleotides as the printing material on the slides.  The hybridization probes consisted of two identical preparations that differed only in their label, so the Cy5/Cy3 ratios were expected to be close to 1. Two different image analysis programs, Scanalyze (Michael Eisen, Stanford) and MAGIC Tool (Davidson College), were used to produce the ratio data.  The observed ratios were generally lower than the expected ratio of 1, due to weaker signal in Cy5.  Although further studies are required to determine the optimal probe to buffer concentrations and the incubation temperature, the signals obtained from the spotted oligos in this study seem sensitive and strong enough to generate quality ratio data.

Introduction

High through-put methods such as DNA microarray technology have evolved to improve its power to determine gene functions in genome-wide scale with incredible speed.  For example, Winzeler et al. employed the “barcode” microarray technology to allow phenotypic analysis of individual gene deletion in yeast, using barcodes to identify their yeast deletion strains.  The power and high speed of the barcode method originates from using one pair of primers, which amplifies all the individual barcodes simultaneously by PCR.  The GeneChip® arrays (Affymetrix Inc.) used in this experiment contained unique barcode (or tag) sequences of 20 bases which were synthesized from the bottom of the glass slides and each base was added upwards.  For each gene knockout, the insert contained two unique barcodes, UPTAG and DOWNTAG, that flanked the marker gene.  The two barcodes for one gene provided an internal control for comparison.  The identified genes essential for viability from this deletion study, especially the ones without human orthologs, have been suggested to be the best targets for antifungal drugs.

In our study, unmodified 40-mer oligonucleotides were directly spotted on the glass sides.  The oligos contained two identical sequences of 20 base pairs, and were not controlled to be oriented in any specific way when spotted.  We were curious to find out the effects of steric hindrance, especially given the short length of our target oligos.  Our study also considered optimizing conditions for this barcode approach such as probe concentration in hybridization buffer, and the temperature setting.  Two different computer softwares, Scanalyze and MAGIC Tool, were employed to analyze the ratio data and raw pixel intensities.  Also, there has been evidence of point mutations in the tags, and we were curious to know how much the mutantions affected the binding capacity of the probes to the barcodes.  This study will serve as a prototype for a grant proposal for a larger project that uses barcodes in Johns Hopkins University.

Printing the barcode microarrays

The 96 samples of 40-mer barcodes were sent to us by Dr. Daniel S. Yuan at Johns Hopkins University School of Medicine.  The frozen oligos were resuspended in deionized water at 100 μM for stock solutions, and the diluted working solutions were spotted on slides.  The optimal final concentration of DNA (48 μM) after resuspension was determined based on an earlier study conducted on 70-mer unmodified oligonucleotides by Wang et al.  An online resuspension calculator was used.  Using the TAS software and MicroGrid II Compact (Biorobotics, Inc) provided by Davidson College biology department, the 96 samples were printed in duplicates in an asymmetrical configuration for easier orientation identification in the images (Figure 1).  The printing pins were thoroughly cleaned following a standard procedure before each printing trial.  On EPO-25C epoxy slides from CEL Associates, Inc., twenty nine slides were printed in total, and each slide contained two metagrids (Figure 2). The spotted arrays were exposed to UV light for 3 minutes for cross-linking, then were washed according to a previously used protocol.

| 70 | 70 | 71 | 71 | 72 | 72 | 73 | 73 |  |  |  |  |  |  |  | 94 | 94 | 95 | 95 | 96 | 96 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65 | 65 | 66 | 66 | 67 | 67 | 68 | 68 | 69 | 69 |  | 89 | 89 | 90 | 90 | 91 | 91 | 92 | 92 | 93 | 93 |
| 60 | 60 | 61 | 61 | 62 | 62 | 63 | 63 | 64 | 64 |  | 84 | 84 | 85 | 85 | 86 | 86 | 87 | 87 | 88 | 88 |
| 55 | 55 | 56 | 56 | 57 | 57 | 58 | 58 | 59 | 59 |  | 79 | 79 | 80 | 80 | 81 | 81 | 82 | 82 | 83 | 83 |
| 50 | 50 | 51 | 51 | 52 | 52 | 53 | 53 | 54 | 54 |  | 74 | 74 | 75 | 75 | 76 | 76 | 77 | 77 | 78 | 78 |
| 21 | 21 | 22 | 22 | 23 | 23 | 24 | 24 |  |  |  | 45 | 45 | 46 | 46 | 47 | 47 | 48 | 48 | 49 | 49 |
| 16 | 16 | 17 | 17 | 18 | 18 | 19 | 19 | 20 | 20 |  | 40 | 40 | 41 | 41 | 42 | 42 | 43 | 43 | 44 | 44 |
| 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 15 | 15 |  | 35 | 35 | 36 | 36 | 37 | 37 | 38 | 38 | 39 | 39 |
| 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 10 | 10 |  | 30 | 30 | 31 | 31 | 32 | 32 | 33 | 33 | 34 | 34 |
| 1 | 1 | 2 | 2 | 3 | 3 | 4 | 4 | 5 | 5 |  | 25 | 25 | 26 | 26 | 27 | 27 | 28 | 28 | 29 | 29 |

Figure 1. The complete layout of the 96 oligos printed in duplicates. This represents one metagrid.

Figure 2. Physical dimensions of one printed slide.  The brackets show where the spots are located.
The figure is not drawn in scale. The edges, slide number and the initials were etched with a diamond tipped pen.  Each slide contained two metegrids.

Hybridization

To detect presence of DNA on the slides, a non-specific dye SYBR Green II (Molecular Probes, Inc) was used on one of the slides (FIGURE 3).  After confirming the presence of the oligonucleotides on the slide, hybridization to the probes was carried out. The probes were already labeled with Cy3 and Cy5, and were sent from Dr. Yuan (labeled: Downtags labeled for hybridization (Cy3, Cy5), D Yuan, 9/3/03).  The dyes were directly incorporated into the PCR products.

1st Trial

The protocol from Dr. Yuan was strictly followed.  Only one slide was used for hybridization.  The incubation temperature was set at 42C, and probe to buffer ratio was 1:500.  Glass coverslips were put over the spot areas, and the slide was put into a blue cap tube wrapped in aluminum foil.  The tube was leveled horizontally, and was put into a dark hybridization chamber for 14 hours.

2nd Trial

Because almost no signal was detected in the 1st trial for Cy5 but slightly more for Cy3, we tried to test higher probe concentrations.  Only one slide was used.  The new probe to buffer concentrations tested were 1:23 and 1:50.  The incubation temperature was set at 37C, to compensate for the melting temperature for shorter oligos.

3rd Trial

The new probe to buffer ratios were 1:10, 1:5, 1:3.3 and 1:2.5.  The incubation temperature was kept at 37C.  Two slides were used.  (Figures 4, 5, 6)

Table 1. The probe to buffer ratios and the respective volumes used for the thrid hybridization trial.

| Probe:Buffer | Probe (l) | Buffer (l) | Total (l) |
| --- | --- | --- | --- |
| 1:10 | 2.5 | 22.5 | 25 |
| 1:5 | 5 | 20 | 25 |
| 1:3.3 | 7.5 | 17.5 | 25 |
| 1:2.5 | 10 | 15 | 25 |

Image Analysis

The microarray chips were scanned using arrayWoRxe biochip reader (arrayWoRxe 2.0 software, Applied Precision, Inc.) provided by Davidson College biology department.  Two different computer programs were used to analyze the tiff images, Scanalyze v. 2.50 (Michael Eisen, Stanford) and MAGIC Tool (Davidson College), to analyze variability in the ratios (Cy5/Cy3) depending on the programs used.  Scanalyze automatically generates many columns of ratios, numbers of counted pixels and intensities calculated in different ways for foreground and background regions after the gridding process.  The columns with uncorrected mean pixel intensities with no background subtraction were used to evaluate the strength of the spot signals, and to compare mean spot intensities for each channel.  MAGIC Tool allows the users to choose many of the gridding and segmentation parameters.  To simulate how Scanalyze generates the ratios (RAT2) as closely as possible using MAGIC Tool, fixed circle method was chosen for segmentation, and average signal with background subtraction was used for generating the ratios.  Fixed circle method assumes that all spots have roughly circular morphology of the same size.  The area outside the fixed circles is considered background, while inside each fixed circle is counted for foreground.  Background subtraction sometimes makes the ratios less accurate due to bright dust spots and artifacts that lie outside the circles.  The total signal option for ratio calculation adds the pixel intensities for all the pixels that lie within a circle and divides that number to the other sum of pixel intensities in the other channel.  The average signal option takes the sum of all the pixel intensities and divides that number by the number of pixels counted for that foreground circle.

Table 2. Comparison of different methods for calculating ratios.

| Methods for calculating ratios | Equations |
| --- | --- |
| Total signal (MAGIC tool) | ## Total foreground intensity in channel 1 Total foreground intensity in channel 2 |
| Average signal (MAGIC tool) | ## Average foreground intensity in channel 1 Average foreground intensity in channel 2 |
| Average – background (MAGIC tool) | ## Average fg in channel 1 – bg in channel 1 Average fg in channel 2 – bg in channel 2 |
| Scanalyze (RAT2 column from spreadsheet) | ## Mean fg in channel 1 – bg in channel 1 ## Mean fg in channel 2 – bg in channel 2 |
| Scanalyze with no background | ## Uncorrected mean intensity in channel 1 Uncorrected mean intensitiy in channel 2 |

Computational Analysis

The 96 oligonucleotide samples included 7 barcodes that should have no signal, 15 pairs of barcodes that have identical tags, and 15 pairs of genes that have different tags (the actual sequence obtained from DNA sequencing and the intended sequence).  However, when I looked at the sequences themselves, I could not find any point mutations in the actual sequences compared to the intended sequences (the pairs were identical to each other for all 15 pairs).  Because the probes provided by Dr. Daniel Yuan consisted of two identical preparations differing only in their Cy5 or Cy3 label, the ratios were expected to be consistent and nominally close to 1.  Uncorrected mean pixel intensities that were outside the boundaries of the average +/- 2*SE were flagged and eliminated, and the 7 genes that should have no signal were all eliminated during this process.  Ratios of 0 or 999 were also considered outliers and were eliminated from computational analysis.  Paired t-test was conducted on the uncorrected mean pixel intensities from Scanalyze to compare the means from the 15 pairs that have different tags to test the null hypothesis that the true mean pixel intensities from the actual sequence tags and the intended sequence tags do not differ from each other.  Another paired t-test was conducted on the 15 pairs of replicates, asking if the mean pixel intensities of the two groups (the originals and the replicates) differed from each other or not.  The ratios generated by Scanalyze and MAGIC tool were compared using ANOVA.  We reasoned that ideally, the three different probe to buffer concentrations tested should give the same ratios.

# Results

SYBR Green II hybridization trial results can be viewed in Figure 3. Results from the third hybridization trial can be viewed in Figures 4, 5 and 6.  These  screen shots are taken from MAGIC tool with the contrast level set on 200%.

Figure 3.  SYBR Green II hybridization to detect presence of DNA on the slide.  This slide was the last slide to be printed, and shows empty places where spots did not print.  But this trial was enough to see that DNA was present on the slides.

Figure 4.  Hybridization result from the third trial. Probe to buffer ratio was 1:5. The spot contrast was set to 200%.

Figure 5.  Result from the third hybridization trial.  Probe to buffer ratio was 1:3.3.  The image contrast was set to 200%.

Figure 6.  Result from the third hybridization trial.  Probe to buffer ratio was 1:2.5.  Image contrast set to 200%.

To analyze the raw pixel intensities, which represent the strength of the signals, for the 15 pairs of replicates, Scanalyze pixel values were examined.  The null oligos that should not give any signal showed pixel intensities that were above background level, but significantly less bright than the rest of the oligos (Table 2.5).

Table 2.5. The mean intensities of the 7 oligos that should not give any signal.  Comparatively, these mean values are less than other mean intensity values reported in Table 3 and 4.

| Mean Cy3 (1:5) | Mean Cy3 (1:3.3) | Mean Cy3 (1:2.5) |
| --- | --- | --- |
| 2350.814536 | 1975.357143 | 2210.714286 |

Because the Cy3 channel gave much stronger signal then Cy5 channel over all, pixel values only from Cy3 channel were evaluated for comparison of signal strength.  Since the spots were printed in duplicates, there were two uncorrected mean pixel values for each spot, and these two values were averaged to represent one barcode.  Paired t-test was conducted to compare the mean uncorrected pixel intensities for the original spot and its corresponding replicate.  The null hypothesis that the two true means were not significantly different could not be rejected for all three different probe: buffer concentrations.

Table 3. Results for paired t-test for the 15 pairs of replicates. The means represent the raw intensities of signals rather than ratios. The null hypothesis is that the two true means are not significantly different from each other.  Degrees of freedom for all three cases was 14.

| ## Probe: Buffer | Meanoriginals | Meanreplicates | t variate | t critical | P value | Reject Null hypothesis? |
| --- | --- | --- | --- | --- | --- | --- |
| 1:5 | 7001.2 | 6303.7 | -0.597 | 2.145 | 0.560 | No |
| 1:3.3 | 3617.9 | 4074.2 | -0.684 | 2.145 | 0.505 | No |
| 1:2.5 | 4709.9 | 3918.4 | 0.964 | 2.145 | 0.352 | No |

In order to assess effects of point mutations, 15 pairs of the actual seqences and the intended sequences were included in the oligonucleotides.  Only Cy3 channel values were used in the same manner as above, and paired t-test was used to compare the two means. The null hypothesis was that the actual sequences did not have better binding capacity than the intended sequences.  For the highest concentration, the null hypothesis was rejected, suggesting that there is a significant possibility that the probes bound better to the actual sequences than the intended.

Table 4. Results for paired t-test for the 15 pairs of actual and intended sequences.  The null hypothesis is that the actual sequences did not have better binding capacity than the intended sequences.  Degrees of freedom for all three cases was 14.  I would like to note that there were no sequence differences between the actual and intended sequences in the sequence spreadsheet.

| ## Probe: Buffer | Cy3 signal Meanactual | Cy3 signal Meanintended | t variate | t critical | P value | Reject Null hypothesis? |
| --- | --- | --- | --- | --- | --- | --- |
| 1:5 | 5215.2 | 4994.1 | 0.42037 | 1.7613 | 0.340 | No |
| 1:3.3 | 3172.7 | 2913.9 | 0.6307 | 1.7613 | 0.269 | No |
| 1:2.5 | 5158.1 | 3463.96 | 3.449 | 2.145 | 0.002 | Yes |

Then instead of the raw pixel intensities, the ratios for the actual and intended sequences were compared for each probe:buffer concentration.  The ratios were generated by Scanalyze, and paired t-test was used to compare the two mean ratios.  The null hypothesis was that the two ratios (one ratio for the actual sequences and the other for the intended sequences) are not different from each other.  For all three different concentrations, the null hypothesis could not be rejected, that there was no significant difference between the means.

Table 4.5. Results for paired t-test for the ratios of actual and intended sequences.  The null hypothesis is that the two ratios are not significantly different from each other.

| Probe: Buffer | Mean ratios for #a | Corresponding barcodes | t variate | t critical | P value | Reject Null hypothesis? |
| --- | --- | --- | --- | --- | --- | --- |
| 1:5 | 0.480073333 | 0.473776667 | 0.33710799 | 2.04840944 | 0.738549 | No |
| 1:3.3 | 0.548166667 | 0.529103333 | 0.83915264 | 2.04840944 | 0.408491 | No |
| 1:2.5 | 0.546023333 | 0.55039 | -0.2253358 | 2.04840944 | 0.823355 | No |

The observed ratios deviated significantly from the expected ratio of 1 in signal comparison for Cy3 and Cy5.  The ratios were skewed towards a value smaller than 1, due to weaker signals in the Cy5 channel.  The ratios produced by a variety of methods are summarized in Table 5.  The means from the three different concentrations within the same method are compared to each other by ANOVA test.  The null hypothesis was that all three means are not significantly different from each other.  When the scanalyze ratios that incorporated background subtraction were tested, the mean ratios hovered around 0.5, and the test gave statistical evidence that the three ratios produced were not equal to each other, suggesting significant variance among the ratios.  When the uncorrected mean pixel intensities from Scanalyze were used to calculate the ratios without background subtraction, the ratios were about 1, but the test gave evidence that the three ratios did not equal to each other.  It could be said that Scanalyze ratio values showed greater variance among the three mean values than the MAGIC tool ratio values.  Although Scanlayze ratios with no background subtraction showed ratios near 1, the variance among the means was too great, rendering the result inconclusive.

Table 5. ANOVA analysis for comparison of three different means. Because the probes provided by Dr. Daniel Yuan consisted of two identical preparations differing only in their Cy5 or Cy3 label, the ratios were expected to be consistent and nominally close to 1.

| Methods | Mean1:5 | Mean1:3.3 | Mean1:2.5 | F variate | F critical | P value | Reject null hypothesis? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total signal (MAGIC tool) | 0.624 | 0.627 | 0.659 | 1.657 | 3.013 | 0.192 | No |
| Total – background (MAGIC tool) | 0.526 | 0.593 | 0.565 | 2.865 | 3.02 | 0.058 | No |
| Scanalyze (RAT2 column from spreadsheet) | 0.463 | 0.526 | 0.540 | 33.75 | 3.012 | 1.593E-14 | Yes |
| Average – background (MAGIC tool) | 0.546 | 0.566 | 0.574 | 3.917 | 3.013 | 0.020 | Yes |
| Scanalyze with no background | 1.332 | 0.924 | 1.033 | 74.87 | 3.013 | 2.27E-29 | Yes |

The ratios generated by the two different programs were compared in Figure 7.  Both sets of ratios incorporated background subtraction.  The intercept was set to 0 since the ratio of 0 should be the same in both programs theoretically.  The slope was 1.1013, showing that MAGIC produced slightly higher ratios than Scanalyze.

Figure 7. Scanalyze ratios are compared to the MAGIC tool ratios.  MAGIC tool used average signal – background to simulate how scanalyze generates its ratios.  The intercept was set to 0.

Discussions

Although further studies are required to determine the optimal probe to buffer concentrations and the incubation temperature, the signals obtained from the spotted oligos in this study seem strong enough to generate quality ratio data.  When the raw pixel intensities were compared for the 15 pairs of replicates, the results demonstrated that there existed a great probability that the 15 pairs of intensity means differed from each other only by chance (Table 3).  Statistically, the compared means did not differ from each other significantly, suggesting robust replicability and validity of using short oligos for microarray printing.  The analysis of the 15 pairs of the actual and the intended sequences seems confounded, because there were no point mutations in the sequences.  The raw intensity of the signals did not show a trend of increase as probe concentration increased (Means in Table 4 and 5).  This lack of trend may be due to the small differences between the concentrations.  Scanalyze ratio values showed greater variance overall than MAGIC tool ratio values, displaying a computer program dependency in the ratio analysis (Table 5).  MAGIC Tool also showed significant variance among the mean ratios, depending on the method chosen in calculating the ratios (Table 5, average – background row, null hypothesis rejected).  It is a standard procedure to use longer exposure time for obtaining Cy5 fluorescence signal, because Cy5 produces weaker signal and degardes more easily than Cy3.  However, longer exposure time for the Cy5 channel means that the background estimation would rise as well.  Thus when background intensities are subtracted from the spot signal, the purpose for the practice of using longer exposure time for Cy5 channel is defeated.  When the Scanalyze ratios were re-calculated without background subtraction with raw intensity scores, the ratios were all near 1 (Table 5, Scanalyze with no background row).  However, generating ratios by this method caused large variances among the means.  It is evident that how the ratios are calculated is just as important as how the data were generated.  Another approach that would lead to stronger signals would be to synthesize the probes that contain dendrimer binding sequences (sticky ends), and use dendrimers for the fluorescent dyes, instead of using probes that directly incorporate Cy3 or Cy5.  The Cy5 dye degrades significantly over time, and since the probes were prepared and left in the freezer in a tube for two months before the hybridization, degradation of probe fluorescence may have been a concern in this study.  Further experiments testing a higher incubation temeprature than 37C would be desirable.  Spotting cDNA or plasmids with inserts for positive controls may also be beneficial for future studies.

# References and Notes

Winzeler EA, Shoemaker DD, Astromoff A, Liang H, Anderson K, Andre B, Bangham   R, Benito R, Boeke JD, Bussey H, Chu AM, Connelly C, Davis K, Dietrich F, Dow SW, El Bakkoury M, Foury F, Friend SH, Gentalen E, Giaever G, Hegemann JH, Jones T, Laub M, Liao H, Davis RW, et al.  Functional characterization of the S. cerevisiae genome by gene deletion and parallel analysis. Science Aug 6 1999 285(5429):901-6.

Winzeler et al.

Affymetrix Inc.  Accessed on Jan 8 2004.

http://www.affymetrix.com/technology/manufacturing/index.affx

Wang HY, Malek RL, Kwitek AE, Greene AS, Luu TV, Behbahani B, Frank B, Quackenbush J, Lee NH. Assessing unmodified 70-mer oligonucleotide probe performance on glass-slide microarrays.  Genome Biol. 2003.  http://www.pubmedcentral.gov/articlerender.fcgi?artid=151289

http://www.idtdna.com/program/techbulletins/Resuspending_Lyophilized_Oligos.asp This web calculator was provided by the same company (Integrated DNA Technologies, Inc) that sequenced the 96 oligos for Dr. Yuan.

Adapted from Oldham, E. E. 2003. Exploring the use of controls in DNA microarrays: a case study in quality control. Soaked pins in 50mM KOH for 5 minutes, careful not to let pin holder touch the liquid. Rinsed pins in dH2O by wiggling in cup for ~5 seconds twice (in 2 separate containers) to wash away excess salt.  Sonicated for 30 minutes to 1 hour.  Soaked in 90-95% EtOH for 30 minutes to 1 hour. Dried the pins by hanging above EtOH for at least 5 minutes.

http://www.cel-1.com

Adapted from Oldham, 2003. Soaked for 2 minutes in 0.1% SDS at room temperature.  Then rinsed in 0.1% SDS for 2 minutes, then in dH2O for 2 minutes at room temperature. Then immersed in boiling water bath for 2.5 minutes. Soaked in 95% EtOH for 1 minute at room temperature.  Blew the liquid off the slides with CO2.

Adapted from Oldham, 2003. Diluted SYBR Green II X10,000 fold. Covered one metagrid with the dilute stain.  Incubated at room temperature for 2~3 minutes without coverslip.  Washed 3~4 times with TBE. Blew the liquid off with CO2. Scanned on channel Alexa flour 488.

SUGGESTED PROTOCOLS FOR HYBRIDIZATION OF SAMPLES

Daniel Yuan, laboratory of Jef Boeke, Johns Hopkins Univ School of Medicine

PREPARATION OF SAMPLES

Storage -- Samples are already labeled with Cy3 and Cy5 dyes. It is

now known that Cy5 is very sensitive to traces of atmospheric ozone,

as well as to light and air (see comment in Science (2003) 301:737 and

description in Anal Chem (2003) 10.1021/ac034241b). Keep samples

frozen under an inert gas until ready for use.

Incubate -- Heat at 72oC for 3 minutes (a PCR program is the easiest way).

Quench in an agitated ice slurry. Keep ice-cold and dark for use.

HYBRIDIZATION

Hybridization buffer -- This is based on Siew-Loon Ooi's

hybridization solution ("SSTE") (Ooi SL, Shoemaker DD, Boeke JD (2001)

Science 294:2552-6). Composition is NaCl 1 M, Tris-Cl 100 mM pH 7.5,

Triton X-100 0.5%. Directions -- Extrude 2.5 ml neat Triton X-100 (use

a batch that is completely colorless) by syringe into ~100 ml

(distilled) water. Microwave to dissolve. Then add water to ~400 ml,

29.2g NaCl and 6.0g Tris base, and titrate with ~ 3 ml concentrated

HCl to pH 7.5. Add water to 500 ml.  WARNING -- It is important to

dissolve the Triton first, because microwaving Triton solutions in the

presence of salt results in an emulsion. Also, direct titration is

required, because Tris buffers appear to become more alkaline when diluted.

DTT -- A 1 molar stock solution of DTT is 154 mg/ml, in water.

Keep this frozen until use.

Incubate -- In a separate tube, combine the sample and warm (42oC)

hybridization buffer in a 1:500 ratio. Add DTT to 1 mM to retard

degradation of Cy5 (Xuewen Pan's suggestion). Incubate in the

hybridization format of choice (e.g., under a glass coverslip, in a

Kapak heat-sealable bag, or in a hybridization chamber) for 12-24 hours at 42oC.

WASHING

20xSSPE stock -- Composition is 3 M NaCl, 200 mM NaH2PO4, 20 mM EDTA,

pH 7.4. See any standard Molecular Biology methods

handbook. Completely dissolve 14.8g EDTA (Na2, 2H20) and 5.2 g NaOH in

~300 ml water before adding 55.2g monobasic phosphate (1H2O), 350g

NaCl, and water to 2 liters. WARNING -- It is important to dissolve

the EDTA first. The EDTA will not dissolve in phosphate buffer, even

as the Na2 salt.

Wash solution -- 6xSSPE with Triton X-100 0.05% (Siew-Loon). Mix 1400

ml water + 600 ml 20xSSPE. Add Triton X-100 freshly (500 ul 10% per

100 ml solution) (10x more than the Winzeler method). Add DTT 1 mM freshly before use.

Hybridization rinse -- 1xSSPE -- 25 ml 20x + water to 500 ml, filtered

through 0.2 um CN. No Triton is added. Add DTT 1 mM freshly before use.

This is also used as a transport buffer.

Wash -- In a 50 ml tube containing 40 ml of Triton wash buffer,

individually manage each slide. Swirl gently and lift the slide out of

the buffer with forceps, drain momentarily, and reimmerse, being sure

to include the entire slide. Do this several times over ~10

minutes. Note any exceptions, since this may be a key determinant of

stringency. Transfer to the Triton-free wash buffer and repeat the procedure.

Scan -- Experiments have shown that signal decreases very slowly

when slides are stored in the Triton-free wash buffer (half-life of

about three days at room temperature), so the buffer can be regarded

as a transport medium until the slides are ready to scan.

Pierce BD, Campbell AM. 2003. Exploring the use of plasmids in DNA microarray technology.